|  | AEROLinkers |  |
| --- | --- | --- |

A Detailed Design REPORT

Submitted by

Mr. S. Nithilan, IV-ECE

Ms. S. Monisa, II-ECE

Mr. T. P. Nishanth, II-ECE

Product 360 Contest

by

VVDN Technologies Private Limited

K.S. RANGASAMY COLLEGE OF TECHNOLOGY

(Autonomous)

TIRUCHENGODE – 637 215

January 2025

Product 360 – Abstract

| Team Name | AeroLinkers |
| --- | --- |
| College Name | K.S.Rangasamy College of Technology |
| Participants Name | Mr. S. Nithilan, IV-ECE |
|  | Ms. S. Monisa, II-ECE |
|  | Mr. T. P. Nishanth, II-ECE |
| Mentor Name | Dr.C.Rajasekaran |
| Document Version | V1.3 |
| Date | 20-01-2026 |

# 1. Architecture (Section 1)

## Architecture Overview and Justification

The system architecture is designed as a low-power, event-driven wearable platform optimized for intermittent connectivity, continuous sensing, and short duration user interactions. The architecture separates always on ultra-low power functions from high energy, event-based operations to achieve predictable battery life and reduced idle power consumption.

A centralized power management subsystem controls multiple regulated domains, enabling selective power gating of high current peripherals.

## Hardware Architecture

The hardware architecture consists of:

- A single ultra-low-power MCU responsible for system control

- A companion Wi-Fi IC activated only during connectivity events

- A Power Management Integrated Circuit with battery charging and regulation

- Dedicated subsystems for display, touch, audio, haptics, and motion sensing

Each hardware block is supplied through controlled power rails and communicates with the MCU using standard digital interfaces.

(Detailed design is covered in Section 2)

## Software Architecture

The firmware architecture employs a layered RTOS-based design with clear separation between boot-time execution, system management services, hardware drivers, and application-level logic. System behaviour is governed by event notifications rather than continuous polling, minimizing CPU wake ups. Peripheral power states are dynamically managed within the scheduler context to reduce idle current and avoid unnecessary resource activation.

The system software is designed to interoperate with the ATAB Server for selected cloud-assisted functions such as configuration synchronization, device registration, and event data exchange. ATAB Server APIs are consumed strictly at the application layer, ensuring that core device functionality remains operational in offline conditions.

The integration follows a client-initiated, event-based communication model, where the device establishes connectivity only when required. No continuous background synchronization is implemented. This design minimizes power consumption and aligns with the wearable’s event-driven architecture.

ATAB APIs are accessed over a secure IP channel using standard request–response semantics. The API layer is isolated from hardware drivers and RTOS services to prevent tight coupling and to allow future server-side evolution without impacting low-level firmware.

(Detailed design is covered in Section 3)

## Mechanical Architecture

The mechanical architecture is designed for a compact wrist-worn form factor, integrating the PCB, battery, display, and haptic components within a sealed enclosure suitable for daily wearable use.

(Detailed design is covered in Section 4)

# 2. Hardware Detailed Design (Section 2)

## Component Selection Matrix

This section documents all key hardware components selected for the system along with justification.

Table 2.1.1 – MCU Comparison and Selection

| Parameter | STM32U375RG (Selected) | nRF5340 | CC3551E | STM32U5 |
| --- | --- | --- | --- | --- |
| MCU type | Ultra-low-power MCU | Radio-centric SoC | Wi-Fi MCU SoC | Feature MCU |
| Integrated radio | None | BLE | Wi-Fi | None |
| Active current @ 64 MHz | ~32 µA/MHz | ~55 µA/MHz | ~48 µA/MHz | ~40 µA/MHz |
| Deep sleep current | ~1.6 µA | ~2.2 µA | ~2.8 µA | ~1.9 µA |
| Wake-up latency | <10 ms | ~15 ms | ~18 ms | ~12 ms |
| Flash | Up to 1 MB | 512 KB–1 MB | 1 MB | 512 KB–2 MB |
| SRAM | Up to 256 KB | 256 KB | 256 KB | 128–786 KB |
| Lead time | 6–8 weeks | 10–12 weeks | 12–14 weeks | 8–10 weeks |

MCU Justification

STM32U375RG MCU

STM32U375RG was selected because it delivers the lowest energy per task, the lowest deep sleep current, and no RF related idle leakage, while providing sufficient on-chip memory and certified hardware security. MCUs with integrated radios were rejected due to higher idle power, RF coexistence risk, and increased firmware complexity unsuitable for an event driven wearable.

Table 2.1.2 – Wi-Fi Solution Comparison

| Parameter | nRF7002 (Selected) | CC3351 | ESP32-C6 |
| --- | --- | --- | --- |
| Wi-Fi architecture | Companion IC | Companion IC | SoC |
| Wi-Fi standard | Wi-Fi 6 (2.4/5 GHz) | Wi-Fi 6 | Wi-Fi 6 |
| Sleep current | <5 µA | ~8 µA | ~20 µA |
| TX peak current | 262 mA | 400 mA | 480 mA |
| Energy per TX event | Lowest | 22% | 45% |
| Interface | SPI | SPI | Internal |
| 10k unit cost | ~$4.8–5.2 | ~$5.6–6.0 | ~$2.8–3.2 |

Wi-Fi Justification

NRF7002 Wi-Fi 6 Companion IC

nRF7002 provides the lowest Wi-Fi energy per event among companion class devices while allowing complete power gating when idle. SoC based Wi-Fi solutions were rejected due to excessive idle current and increased firmware and certification complexity incompatible with smartwatch class power budgets.

Table 2.1.3 – PMIC Comparison

| Parameter | nPM1300 (Selected) | MAX20353 | nPM1304 |
| --- | --- | --- | --- |
| Integrated charger | Yes | Yes | Yes |
| Buck regulators | 1 programmable | 2 (limited) | 1 |
| Max buck voltage | 3.3 V | 3.85 V | 3.3 V |
| Wearable optimization | Yes | Medium | Medium |
| System complexity | Low | Medium | Low |
| Rail flexibility | Limited | Medium | Limited |
| Lead time | 6–8 weeks | 8–10 weeks | 6–8 weeks |

PMIC Justification

NPM1300 PMIC

nPM1300 was selected for its low complexity wearable class integration, combining charging, regulation, and fuel gauging. Although it offers limited rail flexibility, system power domains are consolidated to avoid unnecessary regulators, reducing BOM and integration risk.

Table 2.1.4 – IMU Comparison

| Parameter | LSM6DSOX (Selected) | BMI270 | ICM-42670 |
| --- | --- | --- | --- |
| Accel LP current | ~12 µA | ~14 µA | ~15 µA |
| Normal accel current | ~550 µA | ~685 µA | ~720 µA |
| Embedded intelligence | MLC + FSM | FSM only | None |
| MCU wake reduction | Highest | Medium | Low |
| Energy per wake event | Lowest | +25% | +30% |
| Lead time | 6–8 weeks | 10–12 weeks | 12–14 weeks |

IMU Justification

LSM6DSOX 6-Axis IMU

LSM6DSOX was selected due to its embedded motion intelligence, allowing motion decisions without MCU wake ups, resulting in the lowest always-on power consumption and reduced firmware complexity.

Table 2.1.5 – Audio Codec Comparison

| Parameter | SGTL5000 (Selected) | MAX98090 | TLV320AIC3204 |
| --- | --- | --- | --- |
| Idle | ~6 mA | ~9 mA | ~7 mA |
| Voice playback current | 20–25 mA current | ~30 mA | ~28 mA |

Audio Justification

SGTL5000 Stereo Codec Chip

SGTL5000 was selected for voice-prompt-centric usage, offering the lowest energy per alert and minimal analog and firmware complexity. Music-grade codecs were rejected as over-engineered for Gen-1 wearable requirements.

Table 2.1.6 – Haptic Driver Comparison

| Parameter | DRV2603 (Selected) | DRV2625 | AW8697 |
| --- | --- | --- | --- |
| Motor type | LRA | LRA | LRA + ERM |
| Control | Closed-loop | Closed-loop | Semi-open |
| Idle current | ~0.5 mA | ~1.0 mA | ~1.5 mA |

Haptic Justification

DRV2603 was selected for its voltage independent vibration consistency, lowest idle current, and minimal firmware involvement, making it ideal for accessibility alerts in a wearable device.

Display and Touch Summary

- Display: 1.69″ IPS TFT (280×240)

- Driver: ST7789V2

- Touch: CST816D (single-touch)

This combination provides:

- Predictable power consumption

- Mature driver ecosystem

- Lowest BOM and validation risk for Gen-1

This section describes all digital and analog interfaces.

## Interfaces

The system interfaces are designed to support an event-driven, ultra-low-power wearable architecture while maintaining electrical simplicity, signal integrity, and firmware robustness. All interfaces operate at a nominal logic voltage of 2.7 V, eliminating the need for level translation and reducing static and dynamic power consumption.

High-speed interfaces are limited to short-duration, on-demand activity to minimize EMI and energy loss, while low-speed control interfaces remain idle for the majority of system lifetime. Interface selection prioritizes mature protocols, wide ecosystem support, and deterministic timing behavior, ensuring reliable operation in a compact smartwatch-class form factor.

All interfaces are electrically isolated through power-domain control, ensuring that inactive subsystems do not introduce leakage paths or unintended loading. This interface strategy provides a balanced trade-off between power efficiency, signal integrity, and system scalability,

Table 2.2 – Interface Description

| Interface | Source | Target | Voltage | Frequency / Data Rate | Purpose |
| --- | --- | --- | --- | --- | --- |
| SPI-1 | MCU | IMU | 1.8–3.3 V | Up to 10 MHz | Motion data |
| SPI-2 | MCU | Display | 1.8–3.3 V | 20–40 MHz | UI rendering |
| SPI-QSPI | MCU | Wi-Fi IC | 1.8–3.3 V | upto100 MHz | Network data |
| I²C-1 | MCU | PMIC | 1.8 V | 400 kHz | Power control |
| I²C-2 | MCU | Touch | 3.3 V | 400 kHz | Touch input |
| I²S / SAI | MCU | Audio codec | 1.8–3.3 V | Audio clocks | Voice output |
| GPIO | MCU | Haptic / Button | 3.3 V | — | User feedback |

## DC & AC AnalysiS

### Power Rails – DC Analysis

| Power Rail | Source | Nominal Voltage | Min–Max | Max Load Current | Loads Connected |
| --- | --- | --- | --- | --- | --- |
| VBAT | Li-Po Battery | 3.7 V | 3.0–4.2 V | 800 mA | PMIC |
| VSYS | nPM1300 | Unregulated | VBAT − PMIC drop | 1 A | All rails |
| VOUT-1 | nPM1300 Buck | 2.7 V | 2.62–2.78 V | 200 mA | Display + LED driver |
| VOUT-2 | nPM1300 Buck | 2.7 V | 2.62–2.78 V | 200 mA | Wi-Fi nRF7002 |
| VDD_LDO | TPS76201 | 2.7 V | ±2% | 100 mA | MCU, IMU, Touch, Audio |
| LED Rail | CC Driver | Current mode | — | 40 mA | Backlight |

### MCU GPIO – DC Logic Levels @ 2.7 V

| Parameter | Value | Calculation / Basis |
| --- | --- | --- |
| VDD | 2.7 V | System rail |
| VIH (min) | ≥ 1.9 V | 0.7 × VDD |
| VIL (max) | ≤ 0.8 V | 0.3 × VDD |
| VOH (min) | ≥ 2.5 V | VDD − 0.2 V |
| VOL (max) | ≤ 0.2 V | CMOS spec |
| Max GPIO current | 8 mA | Configured per pin |
| Internal pull-up | 30–50 kΩ | MCU default |

All peripherals selected support 2.7 V logic directly, no level shifting required.

### SPI Interface – DC & AC @ 2.7 V

(Display + Wi-Fi)

| Parameter | SPI-Display | SPI-Wi-Fi |
| --- | --- | --- |
| Logic voltage | 2.7 V | 2.7 V |
| Clock frequency | 40 MHz | 32 MHz |
| VIH requirement | ≥1.9 V | ≥1.9 V |
| Rise/Fall time | <4 ns | <4 ns |
| Trace impedance | ~50 Ω | ~50 Ω |
| Series termination | 22–33 Ω | 33 Ω |
| AC load | Moderate | High (TX bursts) |

AC justification at 2.7 V

- Slightly slower edges vs 3.3 V → lower EMI

- SPI margin remains >25% at 40 MHz with controlled routing

### I²C Interface – DC & AC @ 2.7 V

(IMU, Touch, Audio control)

| Parameter | Value |
| --- | --- |
| Bus voltage | 2.7 V |
| Speed | Fast-mode (400 kHz) |
| Pull-up resistance | 4.7 kΩ |
| Pull-up current | ≈ 0.57 mA (2.7 V / 4.7 kΩ) |
| Max sink capability | >3 mA (all devices) |
| Rise time (tr) | <300 ns |
| Fall time (tf) | <300 ns |
| Bus capacitance | <200 pF |

Pull-up current is intentionally kept <1 mA to reduce static loss.

### I²S Interface – AC Analysis @ 2.7 V

(MCU ↔ SGTL5000)

| Parameter | Value |
| --- | --- |
| Logic voltage | 2.7 V |
| BCLK | 1–3 MHz |
| LRCLK | 16–48 kHz |
| Data width | 16 / 24-bit |
| Clock master | MCU |
| Edge timing | Data valid on rising edge |
| Jitter tolerance | <100 ps |

AC note: Short trace length and reduced swing at 2.7 V significantly lower digital audio EMI.

### Interrupt Lines – DC & AC @ 2.7 V

| Source | Type | Voltage | Trigger | Pull-up | Priority |
| --- | --- | --- | --- | --- | --- |
| User Button | GPIO | 2.7 V | Falling edge | Internal | Level-1 |
| Touch INT | Open-drain | 2.7 V | Falling edge | 10 kΩ | Level-2 |
| IMU INT | Push-pull | 2.7 V | Rising edge | None | Level-3 |
| Wi-Fi IRQ | Push-pull | 2.7 V | Level | None | Level-4 |

### Analog & Mixed-Signal Stability @ 2.7 V

| Subsystem | Sensitivity | Supply | PSRR Needed | Justification |
| --- | --- | --- | --- | --- |
| IMU | High | LDO @ 2.7 V | >60 dB | Motion noise immunity |
| Audio Codec | Very High | LDO @ 2.7 V | >65 dB | Audible noise prevention |
| Touch | Medium | LDO @ 2.7 V | >50 dB | Gesture reliability |
| Display Logic | Low | Buck @ 2.7 V | >40 dB | Digital tolerant |

### EMI & Signal Integrity Summary (2.7 V System)

| Aspect | Implementation |
| --- | --- |
| Reduced voltage swing | Naturally lower EMI |
| High-speed SPI | Series damping + short return loops |
| Audio isolation | Separate LDO + ground island |
| Wi-Fi bursts | Dedicated buck rail |
| ESD | TVS on external interfaces |

### DC & AC Compliance Statement

All digital and analog interfaces operate at 2.7 V nominal, fully within device-specified limits. Signal integrity margins are preserved despite lower swing, while EMI and static power are reduced. This voltage strategy is optimal for an event-driven, Gen-1 wearable and consistent with the locked power architecture.

## Power Sequencing

Power-Up Sequence

- Battery or charger applied to PMIC

- PMIC generates VSYS

- Primary buck enabled (MCU + display)

- MCU boots and configures PMIC

- Secondary rails enabled for audio, IMU, haptic

- Wi-Fi rail enabled only when required

Power-Down Sequence

- Reverse order, Wi-Fi disabled first

## Power Architecture

- Single-cell Li-Po battery

- PMIC buck generates main system rail

- Secondary LDO provides low-noise rail

- Separate power gating for Wi-Fi and backlight

- Battery voltage and current monitored by PMIC

## Power Consumption & Dissipation Calculations

Typical Current Consumption

| Operation (Approx. time) | PMIC and LDO | MCU | Wi-Fi | DISPLAY & Back Light (50% PWM) | IMU | AUDIO | HAPTIC (50% PWM) | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Booting (5 Sec) | 0.823 | 3 | 180 | 50 | 0.55 | 5 | 30 | 269.37 |
| Data Communication (1 Sec) | 0.823 | 3 | 60 | 0 | 0.55 | 5 | 30 | 99.37 |
| Rest Position | 0.823 | 3 | 60 | 0 | 0.55 | 0 | 0 | 94.37 |
| Wake Position | 0.823 | 3 | 60 | 50 | 0.55 | 0 | 0 | 114.37 |
| Docking | 0.823 | 3 | 60 | 0 | 0.55 | 0 | 0 | 64.37 |
| Vibration (1 Sec) | 0.823 | 3 | 60 | 50 | 0.55 | 0 | 30 | 144.37 |

(All values in mA)

Estimated runtime: 18–24 hours typical mixed usage.

## Reset Scheme

- Power-on reset via PMIC

- Software reset controlled by MCU

- Watchdog reset for fault recovery

- User action button triggers application-level reset

## GPIO Mapping

Table 2.8 – GPIO Mapping

| MCU Pin | Signal Name | Direction | Connected Subsystem | Interface | Description |
| --- | --- | --- | --- | --- | --- |
| PB6 | I2C1_SCL | Bi-dir | PMIC (nPM1300) | I2C-1 | PMIC control, charger & fuel-gauge |
| PB3 | I2C1_SDA | Bi-dir | PMIC (nPM1300) | I2C-1 | PMIC control, charger & fuel-gauge |
| PB7 | PMIC_SHPHLD | Output | PMIC | GPIO | Ship-hold / system power latch |
| PC9 | USER_BTN | Input | User Button | GPIO / EXTI | User action interrupt (internal pull-up) |
| PB0 | SPI1_CS | Output | IMU (LSM6DSOX) | SPI-1 | IMU chip select |
| PA7 | SPI1_SCK | Output | IMU (LSM6DSOX) | SPI-1 | IMU SPI clock |
| PA6 | SPI1_MISO | Input | IMU (LSM6DSOX) | SPI-1 | IMU data to MCU |
| PB1 | IMU_INT1 | Input | IMU (LSM6DSOX) | GPIO / EXTI | Motion / wake interrupt |
| PB2 | IMU_INT2 | Input | IMU (LSM6DSOX) | GPIO / EXTI | Secondary motion interrupt |
| PC7 | HAPTIC_EN | Output | Haptic Driver (DRV2603) | GPIO | Enables haptic driver |
| PC8 | HAPTIC_PWM | Output | Haptic Driver (DRV2603) | PWM | Vibration amplitude control |
| PB10 | WIFI_CLK | Output | Wi-Fi (nRF7002) | QSPI-1 | Clock |
| PA4 | WIFI_SS | Output | Wi-Fi (nRF7002) | QSPI-1 | Slave select |
| PC10 | WIFI_D0_MOSI | Output | Wi-Fi (nRF7002) | QSPI-1 | Data 0 / MOSI |
| PC12 | WIFI_D1_MISO | Input | Wi-Fi (nRF7002) | QSPI-1 | Data 1 / MISO |
| PD2 | WIFI_D2 | Bi-dir | Wi-Fi (nRF7002) | QSPI-1 | Data line 2 |
| PC6 | WIFI_D3 | Bi-dir | Wi-Fi (nRF7002) | QSPI-1 | Data line 3 |
| PC5 | WIFI_IRQ | Input | Wi-Fi (nRF7002) | GPIO / EXTI | Wi-Fi interrupt |
| PC0 | I2C3_SCL | Bi-dir | Audio Codec (SGTL5000) | I2C-3 | Codec configuration |
| PC1 | I2C3_SDA | Bi-dir | Audio Codec (SGTL5000) | I2C-3 | Codec configuration |
| PA7 | I2S_DOUT | Input | Audio Codec (SGTL5000) | SAI-1 / I2S | Mic data to MCU |
| PB5 | I2S_DIN | Output | Audio Codec (SGTL5000) | SAI-1 / I2S | Audio data to codec |
| PA5 | I2S_SCLK | Output | Audio Codec (SGTL5000) | SAI-1 / I2S | Serial clock |
| PA4 | I2S_LRCLK | Output | Audio Codec (SGTL5000) | SAI-1 / I2S | Frame sync |
| PC3 | SPI2_DIO | Output | Display (ST7789V2) | SPI-2 | Display data |
| PA9 | SPI2_SCK | Output | Display (ST7789V2) | SPI-2 | Display clock |
| PB12 | SPI2_CS | Output | Display (ST7789V2) | SPI-2 | Display chip select |
| PB13 | TOUCH_I2C_SDA | Bi-dir | Touch (CST816D) | I2C-2 | Touch data |
| PB14 | TOUCH_I2C_SCL | Bi-dir | Touch (CST816D) | I2C-2 | Touch clock |
| PC2 | DISP_RST | Output | Display | GPIO | Display reset |
| PA0 | LED_PWM_BL | Output | Backlight Driver | PWM | LED backlight current control |

## Interrupt Mapping

Priority Order

- User action button

- Touch interrupt

- IMU motion interrupt

- Wi-Fi interrupt

Interrupts are edge-triggered and handled by RTOS ISR callbacks.

# 3. Software Detailed DesigN (Section 3)

## CPU Selection Matrix

(Condensed comparison table retained here, detailed analysis already validated)

STM32U375RG selected due to:

- Lowest µA/MHz

- No RF leakage

- Adequate memory

- Certified security features

## Memory Requirements Analysis & Estimation

This section analyzes the non-volatile and volatile memory requirements of the system firmware and validates that the selected MCU provides sufficient margin for current functionality and future scalability. Memory estimation is based on actual functional blocks, RTOS overhead, and worst-case concurrent execution, rather than optimistic averages.

The system is designed to avoid external memory, reducing power consumption, BOM cost, and integration risk.

### MCU Memory Availability (Reference)

| Memory Type | Available on MCU |
| --- | --- |
| Internal Flash | 512 KB |
| Internal SRAM | 256 KB |
| External Memory | Not used |

### Flash Memory Requirement Analysis

Flash memory is used for bootloader, RTOS kernel, device drivers, application code, configuration tables, and OTA support.

| Flash Usage Category | Estimated Size (KB) | Basis |
| --- | --- | --- |
| Secure Bootloader | 32 KB | Image validation, fallback logic |
| Zephyr RTOS Kernel | 60 KB | Scheduler, IPC, power mgmt |
| HAL & BSP | 45 KB | MCU peripherals |
| Device Drivers | 85 KB | Display, touch, IMU, audio, haptic, PMIC |
| Wi-Fi Stack & Driver | 70 KB | SPI-based companion control |
| Application Logic | 90 KB | UI, alerts, navigation logic |
| Configuration & Tables | 20 KB | Fonts, UI strings, language |
| Diagnostics & CLI | 25 KB | SSH-based CLI |
| Subtotal | 427 KB |  |
| Flash Safety Margin (~15%) | 65 KB | Future updates |
| Total Required | ≈ 492 KB |  |

Conclusion (Flash):

The 512 KB internal flash provides sufficient capacity with controlled margin. Dual-image OTA is possible but constrained; Gen-1 implementation uses single-image + rollback protection to preserve margin.

### RAM Requirement Analysis

RAM usage includes RTOS kernel data, stacks for concurrent threads, heap allocations, DMA buffers, and frame buffers.

a) RTOS & System Overhead

| Component | RAM Usage (KB) |
| --- | --- |
| Zephyr kernel objects | 12 KB |
| IPC queues & events | 8 KB |
| Timer & scheduler data | 6 KB |
| Subtotal | 26 KB |

b) Thread Stack Allocation (Worst-Case)

| Thread / Task | Stack Size (KB) |
| --- | --- |
| Main application thread | 16 KB |
| UI / Display task | 20 KB |
| Wi-Fi control task | 18 KB |
| Audio playback task | 16 KB |
| Sensor / IMU task | 12 KB |
| CLI / Diagnostics task | 14 KB |
| Idle / system task | 6 KB |
| Subtotal | 102 KB |

c) Dynamic Buffers & DMA

| Buffer Type | Size (KB) |
| --- | --- |
| Display line/frame buffers | 32 KB |
| Audio DMA buffers | 16 KB |
| Wi-Fi SPI buffers | 24 KB |
| Sensor data buffers | 8 KB |
| Subtotal | 80 KB |

d) Heap & Safety Margin

| Category | Size (KB) |
| --- | --- |
| Heap allocation | 20 KB |
| RAM safety margin (~10%) | 20 KB |

### Total RAM Estimation Summary

| Category | RAM (KB) |
| --- | --- |
| RTOS & kernel | 26 |
| Thread stacks | 102 |
| Buffers & DMA | 80 |
| Heap | 20 |
| Safety margin | 20 |
| Total Required | 248 KB |

Conclusion (RAM):

The estimated RAM usage fits within the 256 KB internal SRAM with tight but acceptable margin. Stack sizes are intentionally conservative to avoid runtime overflow. No external RAM is required.

### Memory Optimization Strategy

To maintain margin and reliability:

- Display uses line buffering, not full-frame buffering

- Wi-Fi buffers allocated only during active sessions

- Audio buffers released immediately after playback

- Static allocation preferred over heap where possible

- Stack usage monitored using RTOS watermarking

### Memory Sufficiency Conclusion

The selected MCU memory configuration is right-sized for the Gen-1 wearable:

- No excess memory causing leakage

- No external memory integration risk

- Sufficient margin for firmware refinement and feature tuning

This memory strategy aligns with the product goal of low power, low complexity, and high reliability.

## IDE, OS & Device Drivers

- IDE: STM32CubeIDE

- RTOS: Zephyr

- Device drivers: Native Zephyr drivers

- CLI access via SSH using WolfSSH

## Boot Flow

- Power-on reset

- Secure boot validation

- RTOS initialization

- Peripheral bring-up

- Application start

## Key Peripheral Initialization Flow

- Clock configuration

- GPIO setup

- PMIC configuration

- IMU and touch init

- Display init

- Audio and haptic init

## List of Applications

- Device information

- Language selection

- Display settings

- Audio & haptic alerts

- Flight / navigation alerts

- Power and diagnostics

## ATAB Server API Integration

### Purpose of ATAB Server Integration

ATAB Server APIs are used to support non-time-critical, non-safety-critical functions such as:

- Device identification and provisioning

- Retrieval of user- or session-specific data

- Upload of device-generated events (status, alerts)

- Configuration updates

All core wearable functions (navigation cues, alerts, haptics, display, audio) do not depend on continuous ATAB server availability.

### Communication Model

| Aspect | Description |
| --- | --- |
| Communication Type | Client-initiated |
| Transport | IP-based (over Wi-Fi) |
| Session Type | Short-lived |
| Connectivity Mode | Event-triggered |
| Retry Strategy | Limited retries with timeout |
| Offline Behavior | Graceful degradation |

The device initiates communication with ATAB Server only when:

- User action explicitly requires data

- A scheduled sync window is reached

- Configuration refresh is requested

### ATAB API Usage Context

| API Category | Device Usage Context |
| --- | --- |
| Device Registration | First boot / factory reset |
| Device Status API | On-demand health reporting |
| Configuration API | Language, preferences, alerts |
| Event Upload API | Post-event synchronization |
| Authentication APIs | Session establishment |

No ATAB API is invoked from ISR context or real-time tasks.

### Software Layer Placement

| Layer | Responsibility |
| --- | --- |
| Application Layer | API request formation, response parsing |
| Middleware | JSON parsing, message validation |
| Network Stack | TCP/IP, TLS |
| RTOS / HAL | Scheduling, memory, drivers |

ATAB APIs are never accessed directly by device drivers.

### 3.X.5 Security Considerations

- Secure session establishment prior to API access

- Credentials stored in MCU secure storage

- API access restricted to authenticated sessions

- No sensitive data stored persistently unless required

Security implementation aligns with MCU hardware cryptographic capabilities.

### Failure Handling Strategy

| Failure Case | Device Behavior |
| --- | --- |
| Server unreachable | Local operation continues |
| API timeout | Retry with backoff |
| Authentication failure | User notification (if required) |
| Invalid response | Response discarded, logged |

Failure of ATAB APIs does not block user-facing functions.

## Command Line Interface

- Access via Wi-Fi

- SSH-based secure CLI

- Used for diagnostics, logs, and configuration

### ATAB Diagnostic Commands

ATAB integration is supported by CLI-level diagnostics for development and testing purposes.

| Command | Description |
| --- | --- |
| atab_status | Check server connectivity |
| atab_auth | Validate authentication |
| atab_sync | Trigger manual sync |
| atab_log | Display last API transaction |

These commands are disabled or restricted in production builds.

# 4. Mechanical Detailed Design (Section 4)

### 4.1 Exploded View of Concept Design

- Front glass + display

- Touch sensor

- Main PCB

- Battery

- Haptic motor

- Enclosure and strap

Assembly sequence ensures minimal rework and safe battery handling.

Fig 4.1 Exploded view

Fig 4.2 Exploded view in Hand Wrist

GitHub Repository